MIPS Hardware

- different processors understand different patterns (i.e. machine language)

  - a machine language is made up of multiple patterns and each is called a machine instruction

  - many different processor-specific machine languages exist

- in this course, we will use machine language called 32-bit MIPS (Microprocessor Without Interlocked Pipeline Stages)

- below is simplified MIPS processor and RAM (Random Access Memory)

  - Control Unit is brain of processor

    - controls sequence of instructions to execute, interprets the instructions and flow of data, contains the processor’s timing unit, and controls signals to and from peripheral devices

  - Arithmetic Logic Unit (ALU) performs mathematical operations such as addition, multiplication, logical comparison, and decisions requested by control unit

  - general purpose registers each hold 32 bits

    - labelled from $0 to $31

    - not actually general purpose and some have special uses dictated by MIPS

      - $0 is always 0

      - $31 is for return addresses

      - $30 is stack pointer

      - $29 is frame pointer

  - Program Counter (PC) and Instruction Register (IR) are 2 special 32-bit registers used by the Control Unit in fetching and decoding instructions

  - Memory Data Register (MDR) and Memory Address Register (MAR) are used when data travels through data bus

    - when CU wants to read from memory address, it’s stored in the MAR

    - data fetched from address is placed into MDR and then loaded into appropriate register

  - hi and lo registers are used by multiplication and division machine instructions

  - off-chip memory is RAM

    - array of 8-bit slots

    - 4 RAM addresses comprise a single word so we use hexadecimal rep for memory addresses

- to run a program, the program (sitting on hard drive) has to be loaded into memory

  - OS uses special Loader program to do this and we assume it’s loaded at start of memory (i.e. address 0x00)

  - once a program is loaded, memory looks like:

  - loader must do other things to set up other sections of memory for program

    - e.g. loader communicates to processor where it’s loaded the program by setting the Program Counter (PC) register to starting address (0x00 in this case)

- once the loader is done, a hardware-based algorithm called the Fetch-Execute Cycle starts running

  - below is pseudocode of the algorithm

    - infinite loop unless instruction is executed that breaks out of it

    - line 3: reads the machine instruction that starts at memory address PC and stores it into Instruction Register (IR)

    - line 4: all registers and machine instructions are 32 bits and each address of memory is 8 bits, there must be increment of 4 bytes to PC so next instruction can be received

  - this is a program that runs other programs

  - order is important and PC is updated before instruction is actually executed so may affect meaning of instructions that interact with PC

MIPS Machine Language

- below is simplified set of 18 instructions in MIPS machine language:

- add instruction takes values stored in 2 registers (we’ll call them $s and $t) and stores the result in another register $d (i.e. $d = $s + $t)

  - need 5 bits to rep register number since there’s 32 general purpose registers

  - add instruction template is:

  - another format is below, to make relationship with hexadecimal clearer

  - note difference in order that we specify the registers in assembly syntax vs machine syntax

- e.g. write a program in MIPS that takes in the values of registers $5 and $7, adds them together, then stores the result in register $3

  - solution is:

- to negate value, simply subtract it from 0: sub $3, $0, $3

- jump instruction in MIPS changes control flow in program

  - changes PC to value in jump register $s

  - used to exit MIPS program: jr $31

    - $31 is register for return address

    - unlike most HLL where program exits when last instruction is executed, MIPS programs are expected to have an explicit jump to the return address stored by the loader

- bits in registers can be interpreted differently

  - e.g. add and sub instructions interpret them as signed integers but jr instruction interprets them as an address

- to put values (constants called immediates) in registers:

  - stands for load immediate and skip

  - places the next value in RAM into $d and increments the program counter by 4 so that it skips the next line, which is usually not an instruction

  - this places the immediate value we want into the register

- below is a complete example of a MIPS program

MIPS Assembly Language

- assembly language is a textual rep of a machine language

- multiplying two words together might give a word that requires twice as much space

  - to fix this, use registers hi and lo

- mult $s, $t performs multiplication and places the most significant word (4 bytes) in hi and least significant word in lo

  - mult assumes 2 numbers are in signed two’s complement rep but there’s a multu instruction for unsigned ints

  - unless explicitly stated, assume result of multiplying 2 registers is 32-bit value that fits into lo register and doesn’t overflow into hi register

- div $s, $t performs integer division and places the quotient $s / $t in lo and remainder $s % $t in hi

  - sign of remainder matches the sign of the dividend stored in $s

  - divu instructions for unsigned ints

- since results of mult and div are stored in hi and lo registers, there’s special instructions to access the data

- larger amount of memory called Random Access Memory (RAM) is stored off the CPU

  - RAM access is slower than register access but it’s larger

  - data travels between RAM and CPU by bus

  - instructions in RAM start with address 0 and increase by word size (4 bytes)

- each memory block in RAM has an address (one address is one byte)

  - words occur every 4 bytes and are formed from consecutive, (usually) aligned bytes

  - can’t use data directly from RAM, must load into register first

- load word: lw $t, i($s) takes a word from RAM and places it into a register

  - load the word in MEM[$s + i] and store in $t

- store word: sw $t, i($s) takes a word from a register and stores it into RAM

  - load the word in $t and store it in MEM[$s + i]

- the i must be immediate and not another register

Simpler solution:

- MIPS also has control statements

  - beq $s, $t, i means branch on equal

    - if $s == $t, then pc += i * 4

    - i.e. skip ahead i many instructions if $s and $t are equal

  - bne $s, $t, i means branch on not equal

    - if $s != $t, then pc += i * 4

    - i.e. skip ahead i many instructions if $s and $t are not equal

- slt $d, $s, $t means set less than

  - sets value of register $d to be 1 provided the value in register $s is less than the value in register $t and sets it to be 0 otherwise

  - there’s an unsigned version of instruction: sltu

- with branching, we can also do looping since they don’t exist as separate instructions in MIPS assembly language

  - hard-coding –3 could lead to future problems in case we add another line of code in between

- labels are assembler directives and aren’t actually machine code

  - they don’t take words so for bne and beq, labels don’t have line numbers on their own

  - a label at the end of code is allowed and it has the address of the first instruction after the program

- below is same loop using labels:

  - loop has address of 0x14 because they don’t take a word of memory

  - loop is computed as (loop – PC) / 4 where PC is line number after the current line

- MIPS programs can read from standard input using a load instruction to load from the specially designated address 0xFFFF0004

  - loading from this address will read one character from standard input and store it as the LSB within the specific register

- generate output using a store instruction to write to the specially designated address 0xFFFF000C

  - storing at this address will send LSB in specific register to standard output

Implementing Procedures In Assembly Language

- to protect data in registers, preserve register values by saving and storing them in RAM

- register $30 points to the very bottom of free RAM

  - used as a bookmark to separate the used and unused RAM if we allocate from the one end, and push + pop things like a stack

  - $30 is pointer to the top of a stack

  - stack grows from high memory to low memory

    - push to stack by reducing $30 to get lower addresses

    - pop from stack by

- pushing register value on memory stack:

- popping register value off memory stack:

- $30 isn’t stored on the stack but it still gets restored when we reverse the changes we made every time we restore registers from the stack

  - store $1 and $2 in RAM because we need to preserve the values in them before the procedure is run

  - don’t store $3 because the procedure directly modifies the value in it and states that we will place the output value in it

- in order to return to the main program from the procedure, we use jalr which stands for jump and link register

  - sets $31 to be the PC and then sets PC to be $s

  - accomplished by temp = $31, then $31 = PC, then PC = temp

  - since jalr will overwrite $31, first save $31 register to stack

- prologue has all registers that are used by function and stores them in memory stack, body contains all of the function’s code, and epilogue restores all registers from memory stack

  - don’t need to store registers that are being modified by the functions

  - example of calling a procedure from main:

sw $31, -4($30)

lis $31

.word 4

sub $30, $30, $31

lis $1

.word 1234

lis $23

.word SumofDigits

jalr $23

; $2 = 1 + 2 + 3 + 4

lis $25

.word 4

add $30, $30, $25

lw $31, -4($30)

jr $31

; Sums the base ten digits of the

; values of $1. Stores the result in $2.

; Modifies $2

SumofDigits:

; Prologue:

sw $10, -4($30) ;Save $10

sw $3, -8($30) ;Save $3

sw $1, -12($30) ;Save $1

lis $3

.word 12

sub $30, $30, $3

; Body

lis $10

.word 10

add $2, $0, $0

loop:

divu $1, $10

mfhi $3

mflo $1

add $2, $2, $3

bne $1, $0, loop

; Epilogue:

lis $3

.word 12

add $30, $30, $3

lw $10 -4($30)

lw $3 -8($30)

lw $1 -12($30)

jr $31

MIPS Assembler

- any translation process involves 2 phases: analysis and synthesis

  - analysis: understand what’s meant by input source

  - synthesis: outputs the equivalent target code in new format

- input to assembler is text file, which MIPS Assembly language program

  - view it as a string of characters

- first break text file down into meaningful tokens such as labels, numbers, .words, MIPS instructions, etc.

  - done using asm.rkt and asm.cc

  - tokenization is often referred to as Scanning stage

  - if tokens aren’t valid, output ERROR to stderr

  - e.g. loop: add $1, $2, $3 can be converted into stream of tokens: LABEL ID REG COMMA REG COMMA REG

- when writing an assembler, dealing with labels can present issues because labels are assembler directives and don’t exist in machine language

  - assembler must convert each use of label into either an address (if used with .word) or offset (if used with beq or bne)

  - below is 2 valid uses of labels:

    - use of label on right is a problem because it’s used before it’s declared

- solution is to implement the assembler in 2 passes

  - pass 1: scanning and parsing

    - group tokens into instructions and record addresses of labels

    - generate symbol table

    - perform duplicate label checks

    - note that multiple labels are possible for the same line

  - pass 2: semantic analysis and synthesis

    - translate each instruction into machine code

    - if instruction refers to a label, look up associated address and process accordingly

    - check labels that are being used are actually defined

- below is example of symbol table that MIPS assembler would create for the program:

  - when top is used by bne $2, $0, top, assembler must compute offset value

    - offset = (defn − PC) / 4 = (0x14 − 0x20) / 4 = (20 − 32) / 4 = −3

- when writing an assembler:

  - output machine code coming from assembled MIPS code to stdout

  - output symbol table to stderr

- if assembly program is valid, assembler must produce MIPS machine language equivalent on standard output in 2 main steps:

  - first, create encoding for the instructions

  - then, send this output to stdout

- to encode instructions, use bit shifting to put information into correct position, and use bitwise OR to join them

- when dealing with negative values in the offset, use a mask to clear the 16 most significant bits

  - this means applying bitwise AND with 0xFFFF to change the leading 1s to 0s

- below is example of how to encode instructions

  - assuming the assembler has the values of op, s, t, and offset ready:

  - making sure to apply a mask to the –3 in our offset, we can encode the instructions as follows:

- if we wrote cout << instr, then 9 bytes corresponding to the ASCII code for each digit of the instruction would be outputted

  - must use an unsigned char to output the actual bits

- to output machine code instructions:

  - each time, bit shift instr the appropriate amount of places to the right so that the byte we want outputted is in the 8 least significant bits position

  - if concerned about which byte will get copied over when doing the last byte, can mask by doing & 0xFF